# Greenwood catchment study - soil management

1978-1984

Greenwood north of Oakey (QLD)

Keywords:

runoff, erosion, sediment, soil cover, tillage, conservation tillage

# 1. Overview

This catchment study near Greenwood (and sometimes referred to as Acland) was established to examine the influence of soil surface conditions on runoff and soil erosion, on a site representative of one of the major soil types of the eastern Darling Downs.

The study was established in 1980 on a soil representative of one of the major soil types (a brigalow grey clay) on the eastern Darling Downs.  Although runoff leaving a contour bay catchment may entrain material along unstable sections of drainage lines, management at the paddock scale is likely to strongly influence water quality for the whole catchment.

This project examined the influence of rainfall intensity, runoff rate and ground cover on suspended sediment moving through a flume at the exit from a contour bay and the effects of contour banks on sediment delivery to streams were evaluated. Several methods of sampling and calculating suspended soil loss were evaluated.

# Key findings

Surface cover was effective in reducing soil movement and runoff from contour bay catchments. Runoff and sediment concentration were both inversely related to surface cover and total soil movement was greatly reduced by surface cover. Greater than 75% of the variance in soil movement from single events was explained by surface cover and peak runoff rate.

This dataset provides a sound basis for developing and testing water balance/water quality models for cropping and pastures in southern Queensland.

### Observed and predicted mean annual runoff, erosion and water quality at Greenwood (1980/81-1984/5).  Site slope (5%), slope length (35m) LS = 0.6.

| Tillage Management | Observed |  |  |  | Simulated (1980/81-1984/5) |  | Simulated (1900-1999) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Runoff (mm) | Soil erosion (t/ha) | EMC1 (gm/L) | Normalised2 Soil loss (t/ha) | Runoff (mm) | Soil erosion (t/ha) | Runoff (mm) | Soil erosion (t/ha) |
| Stubble burnt | 86 | 38 | 44.2 | 63.3 | 83 | 36 | 49 | 19 |
| Stubble incorporation | 69 | 10 | 14.5 | 13.7 | 71 | 12 | 35 | 5 |
| Stubble mulch | 60 | 5 | 8.3 | 8.3 | 60 | 6 | 31 | 2 |
| Zero tillage | 56 | 3 | 5.4 | 5 | 66 | 3 | 29 | 1 |

1 EMC (event mean concentration) is Soil loss divided by runoff, primary sediment source, no allowance for deposition.

Average annual runoff, soil loss and fallow efficiency for four fallow tillage treatments after a winter crop, and a summer crop-winter fallow (1980-84) (5 fallows) at the Greenwood catchment study. Note: time period where all four tillage treatments have complete data.

Table 1 Influence of tillage and soil cover on runoff and water storage on a grey brigalow clay (Greenwood 1978-83).

| Tillage management (fallow cover) | Bare fallow (<5%) | Disc tillage (25%) | Blade tillage (45%) | Min/no till (>65%) |
| --- | --- | --- | --- | --- |
| Fallow efficiency (%) | 21 | 25 | 26 | 32 |
| Range of FE | 9-29 | 8-38 | 17-36 | 12-32 |
| Reduced runoff (mm) | - | 24 | 32 | 15 |
| Extra soil water (mm) | - | 13 | 16 | 36 |

## Location

Greenwood via Acland, 25 km north-west of Toowoomba.  Lat 27.329, Long 151.726

## Related studies

Greenmount catchment study 1976-2000 Freebairn and Wockner 1986

Wallumbilla catchment study 1982-2000  Freebairn et al 2009

# 2. Experimental conditions (Level 2)

## Site management

The catchment study's configuration was based on an existing soil conservation plan with an established grassed waterway and contour banks (graded) (see aerial photograph above).

While "treatments" were applied, the aim was to create a range of soil surface conditions (soil cover, tillage) by applying five arbitrary tillage/crop regimes. This resulted in soil cover being very low (stubble burnt) to very high (zero tillage), with soil water increasing during summer fallows. A summer crop was grown in one catchment (bay) which has a declined soil water and increasing green cover. Summer fallow treatments were rotated or moved to different bays each year in order to attempt to remove results being attributed to a bay rather than a soil condition. Implicit in this design was that any long term impacts of a "treatment" were ignored or avoided, given the study has no replication and such subtle treatment effects are were unlikely to be observed if they did exist. This was the design philosophy - to create a range of soil conditions, not compare treatments per sec, although data is generally summarised by treatment.

Tillage, planting and harvest operations were carried out by the cooperating farmer (Mr Bill Bosse) with his equipment and occasionally by Queensland Government staff and equipment.

## Treatments

Four catchments were devoted to winter crop (wheat) with intervening summer fallow and a fifth to summer crop. Throughout the summer fallows, a variety of stubble management and fallowing practices provides a range of soil conditions (cover, tillage). Treatments were described as:

- winter crop, stubble burnt, bare soil summer fallow

- winter crop, disc tillage, stubble incorporated summer fallow

- winter crop, blade or chisel tillage, stubble mulched sumerm fallow

- winter crop, no tillage (herbicide weed control), zero tillage summer fallow

- summer crop, chisel tillage, winter fallow

Photograph of tillage/crop treatments

Figure Schematic representation of typical cover distribution for the five cropping-surface management practices applied on the  Greenwood site (Sept.-June). Measured erosion index values are also shown.

Treatment plan for Greenwood, showing rotation of treatments around unreplicated contour bays in order to minimise bias associated with each catchment.

Photograph Runoff and sediment sampling

Schematic of 900 V notch weir, water level recorder and rising stage sediment samplers.

Table  Site characteristics -Greenwood

| Property |  |
| --- | --- |
| Soil type | Grey clay |
| Parent material | Walloon sandstone |
| Catchment area (ha) | 0.72-1.00 |
| Slope (%) | 4-5 |
| Slope length (m) | 35-40 m |
| Channel slope (%) | 0.3 |
| Channel length (m) | 220 |
| Runoff control | Five 900 V notch weirs |
| Water sampling | Rising stage, auto pumping samplers at times |
| Erosion measures | Rill, fans, suspended sediment, bedload at flume or weir |
| Duration | 1978-1984 |

# 3. Project and data management

## Project code:  	 SMPA

Principal investigator: Freebairn D.

Principal data managers: -DM Freebairn GH Wockner

Principal organisations: Queensland Department of Natural Resources, Department of Primary Industries

Data custodian: Queensland Department of Natural Resources

Key co-operators: Mr Bill Bosse

Data access policy: Open access

Planned pathway for data: completed study

Data warehousing: Hydstra, DARTS, this web site

Planned data upload frequency: N/A

Data Quality:		The same two officers collected all the sediment and soil loss data over the entire project duration. The methods never changed and that consistency contributed to making this data set unique.

# Measured data

Rainfall was measured using a network of three tipping bucket recording rain gauges, a manual rain gauge at each side of the catchment, and a gauge at the farm residence approximately 1 km south.

Runoff was measured through recorded gauge height using Cipoletti (trapezoidal) weirs at 5 minute time intervals and summed to daily (9am to 9am) values in this data collection.

Runoff water was sampled using rising stage and in some cased timed samplers at the weir outlet to determine suspended sediment concentrations.

Total soil movement, as described by Freebairn and Wockner (1986a) was measured in this study by estimating the volume of rills and sediment deposits after each major runoff event.

Surface cover (green and dead) was measured after each runoff event, after cultivation operations, at time of soil sampling and periodic site visits.  Expressed as percentage (%) of total soil cover

Surface roughness was described using a semi-quantitative scale (1-5) based on a visual assessment made at the same time as soil cover.

Soil moisture was determined at nine locations in each bay to a depth of 1.5 m in 6 depth increments.  A mean of the nine-samples per depth was used in further analyses.  Soil moisture was determined three times per year; post harvest, mid fallow and pre-planting, typically November, February and May.

Bulk density was measured on several occasions using 10 cm diameter cores, and used to convert gravimetric values to a volumetric basis.

To estimate a PAWC for the site, the mean of the five driest and wettest samplings were used. Expressed as mm available to plants.

Grain yield was determined by harvesting 400 m long strips in each bay using a commercial harvester (6m wide).

Evaporation (E) in fallows was determined by difference as E = Rainfall-Runoff -change in soil water over the fallow.

Fallow efficiency was defined as the change in plant available soil water over a fallow period as a percentage of rainfall. Expressed as percentage (%)

## Data management

A diary of all agronomic operations was kept. See "Other documents' folder.

## Time Series data collected:

Rainfall total and intensity was digitised  from Mort Pluviograph charts and stored in the RRUMS database. This data was later transferred to a Hydstra database and the DARTS database.

Runoff was measured as it discharged into a grassed waterway through  900 V‑notch weirs Water levels were digitised from locally manufactured direct height‑float recorder charts and data transformed and stored in the RRUMS and HYDSTRA databases.

Soil water was measured at the beginning, middle and end of each fallow (summer) period. Soil cores were collected using a hydraulically driven 2.5cm diameter soil core tube. Each soil core was broken in 6 depth increments: 0-10, 10-30, 30-60, 60-90, 90-120 and 120-150 cm. Soil density bulk density was measured from these cores on several occasions.

Soil water lower limit (LL) and drained upper limit (DUL) wer determined from driest and wettest samples respectively over the period of sampling.

Soil cover was measured using 1m^2 quadrants after each runoff event and during site visits (~monthly).

## Suspended Sediment Concentration for Individual Samples:

- Non- sieved or settled samples

This category describes the bulk of the sample analyses previously performed by staff at DNR Resource Management Toowoomba. The procedure is simple enough for untrained personnel to undertake in batches with a minimum of supervision.

- Record sample bottle identification in record book

- Assign and record a beaker for each sample bottle

- Transfer contents of bottle into assigned beaker

- Weigh and record mass of beaker and respective sample

- Add flocculant if necessary, wait for sediment to settle and decant excess water

- Dehydrate beakers at 105° Celsius for no less than 48 hours in suitable dehydrating oven

- Remove beakers from oven, weigh while hot and record

- Discard sediment, wash beakers and re- tare hot if necessary

The procedural aim is to define the respective masses of sediment and water in order to determine the sediment concentration in grams per litre (g.l-1). Masses must be recorded to resolve one hundredth of a gram (0.01g), Ie. grams to two decimal places.

A supply of hot- tared and numbered beakers is set aside specifically for this and other similar procedures. Except for the initial mass of beaker, water and sediment, the masses are recorded hot. This is to ensure that the dry beaker or dry sediment is not permitted to absorb significant quantities of atmospheric moisture. This phenomenon commonly experienced while analysing low sediment concentrations or low masses of sediment in larger surface area receptacles. Simple experiments conducted by staff at DPI Wheat Research Institute (now Leslie Research Centre) indicated that cooling receptacles and sediment could absorb significant quantities of moisture producing incorrect analyses.

## Integrated Suspended Sediment Concentration

Total suspended sediment loss was based on determining a mean sediment concentration for each event:

A mean concentration for rising stage samples collected for each event was determined by flow- weighting the sediment concentration of each stage.

Runoff samples were collected at the weir or flume for suspended sediment determination. Flumes or weirs were installed so that the approach velocity of water was less than 0.04 m.s-1 and a pondage area was constructed upstream from each flume to allow coarse sediment to settle out rather than passing through the structure. This area was cleaned out annually and was large enough to prevent bedload entering the flume on most occasions. This was necessary as H-flumes and V‑notch weirs are not designed to pass bedload without affecting the rating of the structure. A Rising Stage Sampler collected the runoff samples with individual samples collected at 5, 10, 15, 20, 25, 30 and 35 cm stage heights. Each sample was drawn at mid‑flow depth (e.g. the sample taken at 10 cm stage was drawn from 5 cm depth of flow).

The integrated flow‑weighted concentration (CFW) was calculated (Barrett and Loh 1982). It is assumed that the sediment concentration‑discharge relationship for the falling stage is the same as the rising stage and that hydrographs are triangular, i.e. constant rate of rise and fall.

Runoff hydrographs in this study generally had one major peak approximating a triangular hydrograph.  The stage‑concentration relationship of the falling stage is similar to the rising stage, resulting in a representative sample of the whole flow being collected by the rising stage samplers.

For this study, rising stage samplers generally collected representative samples of the whole flow for most events as:

- The majority of bedload material is deposited in the contour channel.

- The bedload component of channel flow was purposefully ‘dropped out’ in the low velocity settling area immediately upstream of the control structure.

- Runoff hydrographs are generally ‘peaky’ and have a short duration due to the small catchment area and high intensity storms.

More detail on water sampling is available in Freebairn and Wockner 1986 Part II

## Event Runoff (mm) (Variable number 766.23)

Surface runoff was measured from each individual catchment through a 900 V-Notch weir.  Water levels were recorded on locally manufactured float recorders.

Construction of contour or graded banks results in major changes in the flow path of runoff. Using the Greenmount site as an example, runoff from the entire paddock would normally flow 360m down a 5 to 7% slope before entering an ephemeral drainage line. When contour banks were installed, flow was divided into six separate hydrologic units, each of 60m at the same slope, with a 250m channel of 0.3% slope leading to a grassed waterway.

Rating information for Greenwood flow-measuring structures:

Flume Type		Calibration (ht is mm GHt)

900 V-Notch		ls = 0.00004375*(ht^2.5); cumecs = ls/1000

The ponded approach area was cleared annually to ensure that the weir rating remained valid  i.e. reduced opportunity for weir being "flooded".

The volume of runoff was calculated using point discharges multiplied by delta time (from previous point to subsequent point ¸ 2). This was divided by catchment area to give runoff in millimetres.

## Event Total Soil Movement (t.ha-1) (variable number 767.23)

The volume of soil deposited in the contour channels was determined after each major erosion event.  The depth and area of each fan was determined by walking through the wet fans and measuring depth of loose soil.  Allowance was made for the depth of cultivated soil.  When the catchments had dried, cross-sections were cut through some fans to check the depth of cultivated and deposited soil against the above measures.  The fan soil density was measured on drying to convert the measured fan volume into a mass value.

This procedure was compared with two other methods:

- Change in channel cross-section was measured after a major event (5 February 1980) at Greenmount (Galletly, personal communication).  The change in cross-section represented 92 t.ha-1 compared to 85 t.ha-1 determined by fan volume.

- Sheet steel plates (10 cm by 10 cm) attached to 20 mm diameter by 30 cm long rods were placed flush with the soil surface where sedimentation was expected (at the base of rill depressions and the ponding areas in front of weirs and flumes) (Lovell, Hairsine pers. comm.).

Soil movement determined using the plates was compared with the survey method in March 1984 (Lovell, personal communication), and measurements of 14 and 15 t/ha were obtained respectively.

Independent operator estimates of soil movement were in agreement, with 10% variation for losses less than 20 t.ha-1 and ± 10 t.ha-1 for losses greater than 50 t.ha-1. Soil movement as estimated by the same two operators throughout the experiment.

In situ soil loss was also measured using steel pins.  Pins (10 mm diameter by 40 cm long) were placed 25 cm into the soil to determine depth of soil removed due to rill and interrill erosion.  They were found to be unsatisfactory due to the need for replacement after cultivation and soil swelling.

## Calculation of Total Soil Movement

Total soil movement is defined as the mass of soil that moves into or through the contour channel.  That is, total soil movement equals deposition in fans plus total suspended load (calculated from Integrated Suspended Sediment Concentration, stored separately as variable 769.23, and Total Runoff, stored as 766.23).  The component deposited in fans is not lost from the paddock, but is removed from the cultivation and crop production plane and deposited in a channel.  When the channel fills with sediment, it must be cleaned out to maintain the performance of the drainage and sediment trap network and also the flow- measuring structures.

Measurement of total soil movement allows data from these experiments to be related to most plot erosion studies where net soil movement to the bottom of the slope is measured. Inter-rill erosion is almost impossible to determine directly from changes in soil level due to this soil’s distinct swelling characteristic.  Therefore the amount of inter-rill erosion was determined by subtracting rill movement from total soil movement.  This analysis allowed the definition of sources and sinks of soil movement within the catchment.  Volumes were converted to masses using measured bulk densities for fans and the cultivated layer.

## Sample management:

Changes during the project: No significant changes to the procedures described in this brief occurred during the lifetime of the project.

Frequency:  Erosion and soil movement were episodic with 84 runoff events occurring over 15 years (5.6 events per year) and rising stage samples were collected on 56 occasions.  When prolonged wet weather and local flooding denied access the rising stage sample on the initial event was used to determine a total sediment loss in the subsequent runoff.  A justification for less rigorous sampling procedures in this type of study could be that suspended sediment represents only 10 to 20% of total soil movement, and is therefore of relatively less importance than other modes of soil movement. If the main objective of a study were to determine accurately total suspended sediment, this simplification would not be appropriate.

Collection time delays: Sample bottles would normally be changed between the first event, and the second compound event, but local flooding could preclude access, demonstrating the limitations of rising stage sampling.

Bottle preparation: Standard 600ml glass "milk bottles" were used for the collection of all rising stage samples. These bottles were thoroughly washed and used as replacement bottles through the life of the project.

# 4. Publications

Ciesiolka, C.C. and D.M. Freebairn, 1982, The influence of scale on runoff and erosion,  Agric. Eng. Conf., Inst. Eng. Aust., Nat. Conf. Publ.,  82/8: 203-206.

Freebairn DM, Wockner GH, Hamilton NA, Rowland P. 2009, Impact of soil conditions on hydrology and water quality for a brown clay in the north eastern cereal zone of Australia. Aust. J. Soil Research, 47 (4):389-402 (http://www.publish.csiro.au/nid/84/issue/5031.htm).

Freebairn, D. M., and Wockner, G. H. (1986). A study of soil erosion on vertisols of the eastern Darling Downs, Queensland. I. Effect of surface conditions on soil movement within contour bay catchments. Aust J. Soil Res. 24, 135‑58.

Freebairn, D. M., and Wockner.  G. H. (1986).  A study of soil erosion on vertisols of the eastern Darling Downs, Queensland. II.  The effect of soil, rainfall and flow conditions on suspended sediment losses. Aust.  J. Soil Res. 24, 159-72.

Freebairn, D., and Boughton, W. C. (1981). Surface runoff experiments in the eastern Darling Downs. Aust. J. Soil Res., 19, 133-46.

# 5. Other documents

data and documents that do not fit into the above

# 6. Climate

Daily weather data was sourced from the SILO database (https://www.longpaddock.qld.gov.au/silo/) and daily rainfall recorded at the site from a digitised record from a Mort Pluviograph. This data was originally stored in the RRUMS database and moved to Project HYDSTRA. Daily rainfall data is available in calendar and .p51 format.

#### Greenwood (Acland) rainfall summary 1976-84

| Water year | Winter rainfall March -October | Summer rainfall Oct-March | % Summer rainfall Oct - March | Water year rainfall (1/10 - 30/9) |
| --- | --- | --- | --- | --- |
| 1976-77 | 108 | 382 | 78 | 490 |
| 1977-78 | 294 | 345 | 54 | 639 |
| 1978-79 | 187 | 408 | 69 | 595 |
| 1979-80 | 115 | 453 | 80 | 568 |
| 1980-81 | 269 | 592 | 69 | 861 |
| 1981-82 | 111 | 473 | 81 | 584 |
| 1982-83 | 460 | 348 | 43 | 808 |
| 1983-84 | 404 | 465 | 53 | 869 |
| Mean 76-84 | 243 | 433 | 59% | 677 |
| Mean (Dalby) 1912-1975 |  |  |  | 673 |

*Water year October to September

# 7. Soil description

The site was selected to represent one of the major soils used for grain production on the Darling Downs. The soil type at Greenwood, a grey clay, Ug 5.16, derived from Walloon sandstone, is a Udic Chromustert and belongs to the Moola clay soil association (Mullins, personal communication).  The profile exhibits a distinct linear gilgai between 10 and 30 cm and shows enhanced concentration of salt at approximately 45 cm.  The soil is of finer surface structure and less strongly self-mulching than the black earth.  The soil is associated with brigalow (Acacia harpophylla) and belah (Casuarina cristata) vegetation. Surface soil properties of the Moola clay at the Greenwood (Acland) experimental sites (0-10 cm depth).

#### Table

#### Soil properties at the Greenwood experimental site

| pH | 8.5 |
| --- | --- |
| CECA (C g -1) | 37 |
| Coarse sand (%) | 10 |
| Fine sand (%) | 24 |
| Silt (%) | 13 |
| Clay (%) | 53 |
| 0. 33 bar water content (g g-1) | 0.35 |
| 1 5 bar water content (g g -1) | 0.20 |
| PAWC B (0 - 150 cm depth) (mm) | 186 |

A Cation exchange capacity.

B Plant available water capacity based on difference between the mean wettest and driest profiles (1978-84).

# 8. Vegetation

# 9. Simulation reports and packages

# 10. Measured Data

Time series of observations are attached. Note format varies between projects.

# 11. Simulations

# 12. Simulation reports, papers