Pasted 4/2/2021

Wallumbilla

1982-2000, Wallumbilla (QLD)

Keywords: rainfall, runoff, erosion, water balance, soil water, sediment, tillage, cover, roughness, wheat, pasture

1. Overview

Introduction

The study was established in 1983 on a soil representative of one of the major soil types (a brigalow brown clay) of the western Darling Downs.  The objective of the study was to explore changes in soil water, catchment hydrology and water quality associated with different soil surface conditions. The site was chosen to represent clay soils with high water holding capacity in the Maranoa region, where fallowing to store water for later crop growth is an essential risk management tool.

Soil water, runoff, and suspended sediment concentrations were monitored on four contour bay catchments over a 18 year period.  A number of different tillage practices and a pasture were used to create a range of soil cover and roughness conditions in order to explore functional relationships between land management, soil conditions, hydrology and water quality. This study aimed to compliment similar studies at Greenmount and Greenwood on the eastern Darling Downs.

Accumulation of soil water in fallows was inefficient, with fallow efficiencies ranging from -7 to 40% due to high evaporation and runoff losses. Runoff amount was determined by soil water content which was strongly influenced by antecedent rainfall, and water use and evaporation patterns. Surface cover and roughness had subtle influences on runoff, and a greater effect on suspended sediment concentrations. Runoff and suspended sediment movement was considerably lower under pasture than cropping.

It is demonstrated that a participative approached between farmers and scientists allowed an extensive long term catchment study to be operated efficiently at a remote location. The data from this study provides benchmark data for future hydrologic and water quality investigations.

Key findings

Surface cover was effective in reducing soil movement and runoff from contour bay catchments. Runoff and sediment concentration were both inversely related to surface cover and total soil movement was greatly reduced by surface cover. Greater than 75% of the variance in soil movement from single events was explained by surface cover and peak runoff rate.

Differences in hydrology and water quality associated with different management (tillage) were more subtle than experienced on the two sites on the eastern Darling Downs (Greenmount and Greenwood), mainly due to the lower levels of stubble available after winter crops due to the drier climate.  Tillage (roughness) was equally influential as cover on runoff.

This dataset has provided a sound basis for developing and testing water balance/water quality models in a contrasting environment to other similar studies.

Table 1: Average Annual Runoff and Soil Erosion for the Wallumbilla study. Howleaky simulation results are presented for trial period and long term annual average.

| Management system | Observed |  |  | Predicted |  | Long term est. (1900-1995) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Cover (%) | Runoff (mm) | Sediment loss (t/ha) | Runoff (mm) | Sediment loss (t/ha) | Runoff (mm) | Sediment loss (t/ha) |
| Chisel tillage (1983-1995) | 10-50 | 53 | 3.1 | 54 | 6.7 | 65 | 6.7 |
| Minimum tillage (1983-1995) | 20-60 | 47 | 2.5 | 50 | 4.4 | 63 | 2.6 |
| Pasture (B condition) (1990-1995) | 50-95 | 9.4 | 0.2 | 13 | 0.5 | 31 | 0.6 |

Annual runoff and suspended sediment loss for 1982/83 to 1994/95 water years for traditional tilled annual cropping at Wallumbilla.

Location:  26o 28'S, 149o 06'E       45 km East North East of Roma in the Maranoa region of southern Queensland.

| 'Fairlands', Wallumbilla (26o 28'S, 149o 06'E), 45 km East North East of Roma in the Maranoa. | Wallumbilla catchment study showing location of weirs (1-4) and soil sampling locations |
| --- | --- |

Related studies

Greenmount catchment study 1976-2000 (Freebairn and Boughton 1981; Freebairn and Wockner 1986)

Greenwood catchment study 1978-84 (Freebairn and Wockner 1986)

2. Experimental conditions

This study  had  four ‘paired’ contour bay catchments and measured catchment hydrology and water quality aspects of different land management options in an actual farm context. Contour bay catchments were chosen as the unit of study as they represent the smallest hydrologic unit that is managed separately, whether this be crop or pasture, tillage or grazing management. This scale required typical farm machinery to be used in implementation of ‘treatments’, applied by the farmer yet still manageable from an instrumentation and logistics viewpoint.

Wheat was sown annually in the crop catchments, planted around May- June depending on planting rainfall, with a median planting date of June 1. Varieties sown were based on time of planting rainfall and regional variety recommendations.  Wheat was typically harvested within two weeks of the beginning of November using the land owner’s harvester. Crops were successfully established in all years, with two seasons with crop failures (1987, 1994).

Pastures were established in Bay 2 in 1990 and Bay 1 in 1996. with Buffel grass (Cenchrus ciliaris) and snail medic (Medicago scutellata) being the predominant species established. Pastures were grazed with cattle when sufficient forage was available and winter crops were not growing (December –March).

Catchment characteristics of the four contour bay catchments at Wallumbilla.

| Bay | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| Area (ha) | 3.7 | 4.0 | 5.9 | 2.4 |
| Slope (%) | 1-3 | 1-3 | 1-2 | 1-3 |
| Slope length (m) | 80-140 | 80-140 | 100-130 | 60-130 |
| Channel length (m) | 460 | 355 | 660 | 290 |
| Establishment date | 14/12/1982 | 14/12/1982 | 14/12/1982 | 13/8/1985 |
| Weir width (m) | 1.216 | 1.216 | 1.216 | 0.914 |
| Period instrumented 1 | 1982-95 | 1982-95 | 1982-95 | 1985-95 |
| 1Reliable records available. |  |  |  |  |

Site characteristics - Wallumbilla

| Property | Description |
| --- | --- |
| Soil type | Brown clay |
| Parent material | Glauconitic lithic sandstone |
| Catchment area (ha) | 2.4-5.9 |
| Slope (%) | 1-3 |
| Slope length (m) | 60-140 |
| Channel slope (%) | 0.22 |
| Channel length (m) | 300-640 |
| Runoff control | Four trapezoidal weirs |
| Water sampling | Rising stage |
| Erosion measures | Suspended sediment |
| Duration of study | 1982-2000 |

Photographs of typical soil conditions created over the duration of the study, resulting in contrasting soil cover and surface roughness (tillage regimes)

and water use patterns (crop and pasture).

Photograph of a Cipoletti or trapezoidal weir with bedload collection area in foreground, (a) water level recorder

and (b).rising stage water sampler

Plan and section of the modified outlet of catchments to: collect bedload and attenuate peak flows

during extreme event to reduce the risk of structure failure.

3. Admin and data management

Project administration

Site identifier code: SMPW (Queensland Govt - Project Hydstra)

Principal investigator: Freebairn DM

Principal data managers: GH Wockner DM Freebairn

Principal organisations: Queensland Department of Natural Resources, Department of Primary Industries

Data custodian: Queensland Department of Natural Resources

Key co-operators: Viv and Barbara Taylor, "Fairlands" Wallumbilla . Rowan and Michael Taylor "Fairlands" Wallumbilla.

Data access policy: Open access. Research has been published (Freebairn et al 2009), base data is available in HYDSTRA (Qld Govt only), DARTS (open access via Qld Globe) and this site.

Planned pathway for data: completed study, Open access

Data warehousing: Project HYDSTRA (Qld Govt only), DARTS (open access via Qld Globe) and this site.

Planned data update frequency: N/A but data may be further refined and error checked.

Data management

Rainfall was measured using a network of three tipping bucket recording rain gauges, a manual rain gauge at each side of the catchment, and a gauge at the farm residence approximately 1 km south. This network provided for some redundancy for equipment failure, and a measure of spatial variability.

A Cipoletti or trapezoidal weir design was chosen for its simplicity of construction and installation, ability to be simply adjusted to match differing catchment areas, and sensitivity to measure total runoff and peak discharge with reasonable resolution (~0.5 mm h-1). Figure in Experimental Conditions section shows a view of a weir, water level recorder and rising stage water sampling equipment installed at all catchment outlets.

Samples of runoff water were collected at the weir outlet to determine suspended sediment concentrations.  Rising stage samples were collected automatically at 5, 10, 20, 30, 40 and 50 cm stage heights with samples being drawn from mid flow depth. Method of calculating a weighted mean sediment concentration is presented in detail by Freebairn and Wockner (1986b), with concentration at each height weighted by its relative discharge at each stage.

Water approach conditions immediately upstream of each weir were modified to; (i) allow bedload to be deposited well before the weir crest ensuring the weir rating remained stable, (ii) allow bedload to be quantified, (iii) reduce backwater effects of the weirs in the channels, and (iv) to provide some protection against overtopping of the weirs in extreme events. Figure in Experimental Conditions shows the changes in design of entrance conditions to each weir. When the water level reaches a critical point, discharge rates are attenuated as water accumulates in the contour channel.  Runoff volume is not affected, but measured peak runoff rates are less than for an uncontrolled outlet. It was recognized from previous experience that large events are responsible for the majority of runoff and soil loss, and it is these extreme events that should be captured without equipment or structure failure.

Water samples collected on the rising limb of hydrographs provide for a robust estimate of total suspended sediment eroded from the hill slope (Freebairn and Wockner, 1986b). Entrainment of ‘suspended’ material from the graded channel was unlikely due to the low channel slope (0.25%) and consequent low stream power of channel flow. The graded channels were typically aggraded with coarse eroded material from the hill slope. The use of rising stage samples as a reliable estimate of total sediment load assumes that the sediment concentration - discharge relationship for the falling stage is similar to the rising. This assumption rarely holds yet Freebairn and Wockner (1986b) demonstrated that this approach provided a pragmatic and reasonable estimate of total suspended load for most events at this scale of study.

Total soil movement, as described by Freebairn and Wockner (1986a) was not measured in this study although that was an original intent.  Rills and sediment deposits were not as distinct as observed in smaller catchments (<1.2 ha), steeper slopes (>5%) and more aggregated soils, making measurement difficult and prone to large errors.

Soil conditions

Surface cover was measured after each runoff event, after cultivation operations, at time of soil sampling and periodic site visits.  Visual estimates of surface cover were made using 1m2 quadrants at 20 random locations in each bay. A diary was kept of all tillage, planting, harvest and spray operations.

Surface roughness was described using a semi-quantitative scale based on a visual assessment made at the same time as soil cover. A smooth surface with no microrelief was rated as roughness 1, while a soil surface with large clods, distinct furrows and maximum potential for surface storage was rated 5. This visual estimate approach was supported by a photographic record.

Soil moisture was determined at nine locations in each bay to a depth of 1.5 m. Soil samples were collected using a hydraulically driven 30 mm diameter steel tube. Each core was separated into intervals of 1-10, 10-30, 30-60, 60-90, 90-120 and 120-150 cm and moisture content determine gravimetrically. A mean of the nine-samples per depth was used in further analyses.  Bulk density was measured on several occasions using 10 cm diameter cores, and used to convert gravimetric values to a volumetric basis. To determine a value for driest and wettest soil moisture at each depth, and to estimate a PAWC for the site, the mean of the five driest and wettest samplings were used. Soil moisture was determined three times per year; post harvest, mid fallow and pre-planting, typically November, February and May.

Other measurements

Grain yield was determined by harvesting 400 m long strips in each bay using a commercial harvester (6m wide). It was recognized that this unreplicated study could not be used to make inferences about yields associated with different practices but provided an indication of the sites productivity.

Evaporation (E) in fallows was determined by difference where E = Rainfall-Runoff -change in soil water over the fallow. Drainage was assumed to be negligible for the purpose of soil water calculations but detailed analysis of chloride distribution have provided estimates of this water pathway, being in the order of 5-15 mm yr-1 (Tolmie et al. 2003). Water use by weeds was ignored, although this was clearly a significant loss mechanism in some seasons when weed control was less than optimum.

Fallow efficiency is defined as the change in plant available soil water over a fallow period as a percentage of rainfall. Soil moisture was measured at the beginning, middle and end of fallows.

Changes during the project:

No significant changes to the procedures described in this brief occurred during the project.

Project Hydstra brief

Sites:  These four sites are shared between several projects. Subsequent to this study this particular series of catchments was been monitored under the “Fate of Nutrients” project (Geoff Titmarsh) although the station numbers remain constant and common to both projects.

AB42216           Wallumbilla  Contour Bay 1

AB42217           Wallumbilla  Contour Bay 2

AB42218           Wallumbilla  Contour Bay 3

AB42219           Wallumbilla  Contour Bay 4

Sample management

Frequency:  Erosion and soil movement were episodic. Over a 13 year period, 66 rainfall events resulted in runoff (Av 5.1 events per year), similar to 14 years/81 events on the eastern Darling Downs (Greenmount - Av 5.7 events per year) despite the difference in mean rainfall at Greenmount being 724 mm compared to Wallumbilla's 587 mm. Rising stage samples were also collected at the outlet of each bay. Higher runoff under the scarified treatment compared with chisel reflects the decreased surface detention and rapid stubble breakdown from tined cultivation.  Minimum tillage with little surface roughness resulted in the most runoff in most years and 'blade tillage' the least.  High runoff under minimum tillage is attributed to the rapid formation of a surface seal reflecting the hard setting nature of the soil.  Tillage by chisel or blade plows reformed surface roughness and broke any surface seal, yet maintained surface cover.  A statistical analysis of runoff indicated that roughness was as important as cover on this soil in terms of improving infiltration.If the main objective of a study were to determine accurately total suspended sediment, this simplification would not be appropriate

Collection time delays:   Sediment sample bottles would normally be changed within 2 days of an event, , but local flooding could preclude access for up to 5 days.

Bottle preparation: Standard 600ml glass milk bottles were used for the collection of all rising stage samples. These bottles were thoroughly washed and used as replacement bottles through the life of the project.

The following estimated parameters are stored as comments with the sample master record: Event rills     (t.ha-1); Event sediment fans    (t.ha-1)

NB. Event = Generally same day but for long events may continue for several days.

Data Type:

Integrated (Flow- Weighted Mean) Suspended Sediment Concentration. (769.23)

Samples of runoff water were collected at the weir outlet for determination of suspended sediment.  Samples were collected by a rising stage sediment sampler.  Methods of calculating suspended loads are presented in detail by Freebairn and Wockner (1986).  While approach conditions to all sampling points were similar, it is likely that the size of ‘suspended’ material would be dependent on discharge rates to some extent. Nevertheless, the samples collected provide a robust estimate of the influence of rainfall and soil conditions on suspended sediment. Entrainment of ‘suspended’ material from the graded channel was unlikely due to the low slope (0.25%) and consequent low stream power of channel flow. The graded channels were typically aggraded with eroded material from the plane.

Suspended Sediment Concentration for Individual Samples:

This procedure is not NATA accredited.

Non- settled samples

This category describes the bulk of the sample analyses previously performed by staff at DNR Resource Management Toowoomba. The procedure is simple enough for untrained personnel to undertake in batches with a minimum of supervision.

Record sample bottle identification in record book

Assign and record a beaker for each sample bottle

Transfer contents of bottle into assigned beaker

Weigh and record mass of beaker and respective sample

Add flocculant if necessary, wait for sediment to settle and decant excess water

Dehydrate beakers at 105° Celsius for no less than 48 hours in suitable dehydrating oven

Remove beakers from oven, weigh while hot and record

Discard sediment, wash beakers and re- tare hot if necessary

The procedural aim is to define the respective masses of sediment and water in order to determine the sediment concentration in grams per liter (g.l-1). Masses must be recorded to resolve one hundredth of a gram (0.01g), Ie. grams to two decimal places.

A supply of hot- tared and numbered beakers is set aside specifically for this and other similar procedures. Except for the initial mass of beaker, water and sediment, the masses are recorded hot. This is to ensure that the dry beaker or dry sediment is not permitted to absorb significant quantities of atmospheric moisture. This phenomenon commonly experienced while analysing low sediment concentrations or low masses of sediment in larger surface area receptacles. Simple experiments conducted by staff at DPI Wheat Research Institute (now Leslie Research Centre) indicated that cooling receptacles and sediment could absorb significant quantities of moisture producing incorrect analyses.

Integrated (Flow- Weighted) Concentration

Total sediment loss was based on determining a mean sediment concentration for each event:

A mean concentration of stage samples collected for each event was determined by flow- weighting the sediment concentration of each stage.

Samples of runoff water were collected at the weirs for determination of suspended sediment.  Weirs were installed so that the approach velocity of water was less than 0.04 m.s-1. A pondage area was constructed upstream from each flume to allow coarse sediment to settle out rather than passing through the structure. This area was cleaned out between events and was large enough to prevent bedload entering the flume on most occasions. This was necessary as trapezoidal weirs are not designed to pass bedload without changing the rating of the structure. Samples of runoff were collected by a rising stage sediment sampler. The rising stage samples were collected at 2.5, 5, 10, 15, 20, 25, and 30 cm stage heights. Each sample was drawn at mid‑flow depth (e.g. the sample taken at 10 cm stage was drawn from 5 cm depth of flow etc.).

The flow‑weighted concentration (CFW) was calculated (Barrett and Loh 1982):

In the use of this method, it is assumed that the sediment concentration‑discharge relationship for the falling stage is the same as the rising stage and that hydrographs are triangular, i.e. constant rate of rise and fall.

Runoff hydrographs in this study generally had one major peak approximating a triangular hydrograph.  The stage‑concentration pattern of the falling stage is similar to the rising stage, resulting in a representative sample of the whole flow being collected by the rising stage samplers.

For this study it was not surprising that rising stage samplers collected representative samples of the whole flow for most events as:

The majority of bedload material is deposited in the contour channel.

The bedload component of channel flow was purposefully ‘dropped out’ in the low velocity settling area immediately upstream of the control structure.

Runoff hydrographs are generally ‘peaky’ and have a short duration due to the small catchment area and high intensity storms.

Event runoff in mm (766.23)

Surface runoff.  Runoff was measured from each catchment as it discharged into a grassed waterway through 0.914 -1.216 m Cipoletti weirs.  Water levels were recorded on locally manufactured direct height float recorders.

Soil movement, runoff and moisture accumulation were measured within each of four contour bay catchments which vary in size from 2.4 to 5.9 ha. Construction of contour or graded banks results in major changes in the flow path of runoff. At the Wallumbilla site, runoff from the paddock would flow 60-140m down a 1 to 2% slope before entering a formed channel. 300m to 638m long of 0.22% slope, leading to a grassed waterway.

Calibration data for Wallumbilla weirs:

Bay GS Area (Ha) Flume Type

1 AB42216 3.7 1.216 m Cipoletti weir

2 AB42217 4.0 1.216 m Cipoletti weir

3 AB42218 5.9 1.216 m Cipoletti weir

4 AB42219 2.4 0.914 m Cipoletti weir

Flume Type : Cipoletti wier  (trapezoidal)                Calibration (ht is mm GHt)

The (theoretical) calibration equations for Wallumbilla from the HYDRO ini file; ("ht" is in millimeters and cumecs are cubic meters per second). Ponded approach area were cleared annually to ensure valid flume rating.

1.216 m weirs (4') Cipoletti Wall Bay 1,2,3  :cumecs = 0.0000716*(ht^1.5)

0.914 m weir  (3') Cipoletti Wall Bay 4      :cumecs = 0.0000537*(ht^1.5)

Total flows calculated using point discharges multiplied by delta time (from previous point to subsequent point)/2.  This was divided by catchment area to give runoff in millimeters.

Event total soil movement in tonnes per hectare. (767.23)

The volume of soil deposited in the contour channels was determined after each major erosion event.  The depth and area of each fan was determined by walking through the wet fans and measuring depth of loose soil.  Allowance was made for the depth of cultivated soil.  Soil density of fans was measured on drying to convert measured volumes to masses.

This procedure was compared with two other methods:

Change in channel cross-section was measured after a major event (5 February 1980) at Greenmount (Galletly, personal communication).  The change in cross-section represented 92 t.ha-1 compared to 85 t.ha-1 determined by fan volume.

Sheet steel plates (10 cm by 10 cm) attached to 20 mm diameter by 30 cm long rods were placed flush with the soil surface at Greenmount where sedimentation was expected (at the base of rill depressions and the ponding areas in front of weirs and flumes).

Soil movement determined using the plates was compared with the survey method in March 1984 (Lovell, personal communication), and measurements of 14 and 15 t.ha-1 were obtained respectively.

Calculation of Total Soil Movement

Total soil movement is defined as the mass of soil that moves into or through the contour channel.  That is, total soil movement equals deposition in fans plus total suspended load (calculated from rising stage samples – stored separately as variable 769.23).  The component deposited in fans is not lost from the paddock, but is removed from the cultivation and crop production plane and deposited in a channel.  When the channel fills with sediment, it must be cleaned out to maintain the performance of the drainage and sediment trap network.

Measurement of total soil movement allows data from these experiments to be related to many plot erosion studies where net soil movement to the bottom of the slope is measured.  Volumes were converted to masses using measured bulk densities for fans and the cultivated layer.

Time Series data collected:

Rainfall total and intensity was determined from digitised Mort Pluviograph charts and stored in the RRUMS database.

Runoff was measured as it discharged into a grassed waterway through 0.914 m - 1.216 m Cipoletti weirs. Water levels were digitised from locally manufactured direct height‑float recorder charts and likewise stored in the RRUMS database.

The conversion process from RRUMS to HYDSYS/TS is described in an associated document:  \\thor\public\hydsys_docs\RRUMS_Conversion\ReadmeRrumsSource.doc

Laboratory: Toowoomba DNR laboratory

| Variable number | Bottle | Parameter | Analysis Method | Reference Code | Min Reporting Level | Units |
| --- | --- | --- | --- | --- | --- | --- |
| 766.23 | IS | Total Runoff | Float recorder and rated structure | DPI | 0.1 | mm |
| 767.23 | IS | Total Soil Movement | Freebairn and Wockner | DPI | 1 | t.ha-1 |
| 769.23 | IS | Integrated Suspended Sediment Concentration | Gravimetric flow- weighted mean | DPI | 100 | mg.l-1 |

4. Climate

Rainfall in the region is highly variable with an annual coefficient of variation of 30%. Over the 18 year period (1983-2000) annual rainfall varied between 329 and 869 mm with a mean of 577 mm, 70% falling in the October-March period. Long term (1900-2000) annual rainfall is 584 mm.

Summer rainfall is characterised by intense convective storms and long dry periods with high evaporative demand. Average annual evaporation (Class A pan) is 2000 mm with maximum daily values reaching 15 mm.  Maximum temperatures in summer frequently exceed 38°C while frosts are common in winter months, restricting planting dates to avoid frost at anthesis.

Summary of annual and seasonal rainfall, Wallumbilla

| Water Year | Oct - March rainfall | Annual rain- (Oct-Mar) | Annual rainfall | % Summer rainfall |
| --- | --- | --- | --- | --- |
| 1982-83 | 246 | 426 | 672 | 37 |
| 1983-84 | 505 | 340 | 845 | 60 |
| 1984-85 | 379 | 173 | 552 | 69 |
| 1985-86 | 318 | 150 | 468 | 68 |
| 1986-87 | 306 | 71 | 377 | 81 |
| 1987-88 | 387 | 252 | 639 | 61 |
| 1988-89 | 465 | 404 | 869 | 54 |
| 1989-90 | 456 | 215 | 671 | 68 |
| 1990-91 | 251 | 78 | 329 | 76 |
| 1991-92 | 333 | 170 | 503 | 66 |
| 1992-93 | 165 | 178 | 343 | 48 |
| 1993-94 | 564 | 39 | 603 | 94 |
| 1994-95 | 400 | 132 | 532 | 75 |
| 1995-96 | 514 | 218 | 732 | 70 |
| 1996-97 | 337 | 92 | 429 | 79 |
| 1997-98 | 376 | 190 | 566 | 66 |
| 1998-99 | 404 | 114 | 518 | 78 |
| 1999-200 | 598 | 140 | 738 | 81 |
| Mean 1982-2000 | 389 |  | 577 | 68 |
| Long term mean (1890-2005) | 391 |  | 560 | 70 |

Monthly average mimimum and maximum temperature, pan evaporation and rainfall  (study period 1983-2000 and long term 1900-2000)

"Fairlands" "Fairlands" Wallumbilla actual trial site rainfall with BoM other meteorology data

Wallumbilla PO Wallumbilla long term climate data from SILO

5. Soil description

The catchment site is 45 km East North East of Roma in the Maranoa region.  The sloping (2%) brown clay soil supported brigalow (Acacia harpophylla)  and belah (Casuarina cristata)  vegetation prior to clearing in 1963 and has been cropped continuously since 1966 (initial crop Mendos wheat 5.4 t/ha).  Salts are concentrated at 40-50 cm, and 80% of potential plant available water (100 mm) is held in the surface 60 cm.

In its virgin state, the soil is most closely related to a Red Brown Earth derived from Glauconitic lithic sandstone and is a Dr (see Robin).  Once cultivated the mixing of the original surface results in the soil being described a brown clay Ug 5.32 (Northcote,1979).  The surface varies from weakly self-mulching to moderately crusted to weakly hardsetting depending on environmental conditions including recent rainfall and cover, and exhibits cracking and self-mulching characteristics when dry.  The soil has a plant water capacity of 150 mm in the top 1.5 m of soil.

Infiltration rate is very high when the soil is dry and cracked.  However, closure of the cracks through swelling and formation of surface seals due to instability of the soil under rainfall reduces infiltration rates (Smith et al.  1984).

As a result of the intensive sampling program over 17 years at this site, temporal variability associated with crops, fallow period and permanent pastures provides a robust estimate of a practical field based Plant Available Water capacity (PAWC). Additionally, with nine sample locations within each catchment, a picture of spatial variability of Plant Available Water (PAW), a system status property, and PAWC, a system desription, within and between catchments has been explored. see Spatial variability in PAW and PAWC

Soil description

(Brian Slater, DPI Roma)

Location: 'Fairlands', Wallumbilla  Qld.  26o32'E  149o 7'S

Great Soil Group: Brown clay  PPF:  Ug 5.32

Landform: Mid/upper slope of gently undulating plain  2% slope 170o aspect

Parental Material: Glauconitic lithic sandstone

Minmi Member of Bungil Formation

(Lower Cretaceous)

Vegetation: Cleared and cultivated at site. Adjacent is open forest disturbed by extensive clearing.

Tallest stratum:   Acacia harpophylla (brigalow)

Eucalyptus populnea (poplar box) and

Casuarina cristata (belah) 60-70% canopy cover, height to 14 m

Midstratum: Eremophila mitchellii (false sandalwood), Geijera parviflora (wilga)

Surface condition: Recently cultivated.  Surface varies from weakly self-mulching to moderately crusted to weakly hard-setting depending on environmental conditions including recent  rainfall and cover.

Soil profile of a brown sodosol

Driest and wettest samples over a 17 year period at Wallumbilla, showing variability in "apparent" PAWC of 180 to 277 mm

Profile description

| Horizon | Depth (m) | Description |
| --- | --- | --- |
| Ap | 0 - 0.15 | Dark brown (7.5 yr 3/3 m); light medium clay; moderate 10-20 mm angular blocky; dry; moderately firm. Clear to - |
| B21 | 0.15-0.28 | Brown (7.5 yr 4/3 m); medium clay; moderate 20-50 mm angular blocky; moderately moist, very firm. Gradual to - |
| B22k | 0.28-0.58 | Dull reddish brown (5 yr 4/4 m); medium clay; moderate 20-50 mm angular blocky and lenticular; moderately moist, moderately strong; few medium calcareous soft segregations; very few fine calcareous nodules. Gradual to - |
| B23y | 0.58-1.06 | Reddish brown (5 yr 4/6); medium clay; moderate 20-50 mm lenticular; moderately moist, moderately strong; many medium gypseous crystals; common coarse gypseous crystals. Gradual to - |
| B24 | 1.06-1.35 | Bright reddish brown (5 yr 5/6); medium clay; moderate 20-50 mm prismatic; dry, moderately strong. Gradual to - |
| BC | 1.35-1.50 | Clay as for B24 mixed with increasing amounts of weathered sandstone. |

Soil Taxonomy classification: Entic Chromustert, mixed, fine, thermic.

FAO UNESCO soil unit: Chromic Vertisol.

US Taxonomy: Vertisols with alkaline horizons over strongly acid clays are rather unique to Brigalow areas of Australia.

Entic chromustert, fine, mixed, thermic.

| Depth | pH |  | EC m S/cm | Cl % | Particle size |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1:5 | CaCl2 |  |  | CS | FS | S | Clay |
| 10 30 60 90 120 150 | 7.9 8.8 8.0 6.1 4.7 4.8 | 7.1 7.9 7.8 5.9 4.4 4.1 | 0.11 0.52 3.50 3.90 1.70 0.72 | 0.003 0.029 0.095 0.105 0.105 0.071 | 6 7 7 7 5 18 | 42 41 42 43 44 55 | 4 5 5 7 8 3 | 44 47 45 43 43 21 |
| Depth | m equiv/100 g soil |  |  |  |  | % Total elements |  |  |
|  | CEC | Ca++ | Mg++ | Na+ | K+ | P | K | S |
| 10 30 60 90 120 150 | 34 31 26 26 28 15 | 17.0 14.0 13.0 11.0 8.4 3.5 | 9.4 13.0 13.0 13.0 12.0 5.9 | 1.4 3.7 4.9 5.3 5.3 2.6 | 0.89 0.32 0.26 0.29 0.29 0.17 | 0.02 0.02 0.01 0.01 0.02 0.02 | 0.68 0.61 0.57 0.69 1.04 1.20 | 0.03 0.09 1.68 2.53 0.70 0.17 |

Brown sodosol pasture Brown sodosol Reference soil description - pasture catchment Wallumbilla catchment study Best bet PAW description based on new LL and density from base file (.xls) CN of 85 and CN cover response of 0 best good fit K factor 0.4 for the site, Delivery ratio of 1.00 - low sediment from hill slope, no observable silt fans, assume all suspended sediment delivered Freebairn et al 2011, Aust. J Soil Research DMF 24/2/2018, 15/1/2021

Brown sodosol Brown Soodosol Reference soil description - cropped catchment Wallumbilla catchment study Best bet PAW description based on new LL and density from base file (.xls) CN of 85 and CN cover response of 10 best good fit K factor 0.4 for the site, Delivery ratio 0.4 - larger catchment, higher channel transport resulting in higher proportion being fines Freebairn et al 2011, Aust. J Soil Research DMF 24/2/2018, 15/1/2021

6. Vegetation

Vegetation (crop or pasture) is described in Howleaky in two modes: Leaf Are (LAI) or  Cover models

An LAI model (from the EPIC model) specifies a LAI development pattern, heat sum development and yield based on water and heat stress. An LAI model is dynamic, driven by daily environmental conditions. While being more "realistic" compared to a Cover model, the model requires some crop physiology expertise to parameterise.

A Cover model specified a vegetation by a time series of green (growing) and dead (crop residue) soil cover. Green cover specifies a transpiration pattern while residue cover specifies soil conditions resulting from tillage and residue management.  This information can be easily sourced from local agronomic expertise.

Total cover moderates evaporation, infiltration and erosion in both model styles

This project includes three specified cover models: chisel tillage and minimum tillage after wheat harvest and a pasture and one LAI model with fixed planting date

Wheat 25 May Generic wheat crop tuned for Wallumbilla Catchment study observations Planting date 25th May, mid-range of actual planting Harvest dates are within 14 days of field harvest Model yields are approximately 18% > than field harvests D Freebairn 15th November 2011, 2021

Pasture good Well managed pasture Buffel grass with winter pasture legumes in some seasons, depending on suitably moist autumn conditions

Wheat-min till Aim for maximum retention of stubble cover, but sometimes limited due to poor herbicide weed control and lack of herbicide application equipment

Wheat-scarify Aim for district practice - little attention to stubble retention

7. Tillage

When the site was initiated in 1982, the intention was to impliment similar "treatments" to those applied at the Greenmount and Greenwood sites.

Being four hours travel from Toowoomba (base for experimental scientists), treatments were implimented by the cooperator Mr Viv Taylor and his family, supported by Mr Gus hamilton and Phil Rowland at the Roma DPI office.

Before each summer season, a concensus was achieved between the site team (Toowoomba, Roa DPI and the farmer) to apply tillage regimes that resulted in maximising differnces between catchment conditions.  Stubble could not be burnt as there was typicvally insufficient stubble to carry a fire. Zero tillage was not practical in most seasons becasue of spray equipment not being readily available to effectively contro, weeds when they were small. Therefore, the soil conditions between "conventional" tillage and c"onservation tillage" were often subtle.  While observations are reported under the tqo treatments and a pasture, the differnecs bewteen treatment swas regarded as unlikely to be measurable efffective.

Two tillage regimes are included for completeness but we consider they should be regarded more as duplicates than teratments.

Chisel

Minimum as per 1111129 HL assessment doc

Planting

8. Measured Data

The aim of observations and measurements at the Wallumbilla catchment study ( 1982-2000) was to quantify major components of the water balance in order to explore interactions between soil conditions (moisture, cover, roughness) and water flow and soil and sediment movement. A range of tillage/crop management practices were applied to create a range of soil conditions.

Rainfall was measured using a network of three tipping bucket recording rain gauges, a manual rain gauge at each side of the catchment, and a gauge at the farm residence approximately 1 km south.

Runoff was measured through recorded gauge heights using Cipolletti (trapezoidal) weirs at 5 minute time intervals and summed to daily (9am to 9am) values in this data collection

Runoff water was sampled using rising stage and in some cases timed samplers at the weir outlet to determine suspended sediment concentrations.

Total soil movement, as described by Freebairn and Wockner (1986a) was not measured in this study as rills and sediment deposits were not as distinct as observed in smaller catchments (<1.2 ha), steeper slopes (>5%) and more aggregated soils.

Surface cover (green and dead) was measured after each runoff event, after cultivation operations, at time of soil sampling and periodic site visits.  Expressed as percentage (%) of total soil cover

Surface roughness was described using a semi-quantitative scale (1-5) based on a visual assessment made at the same time as soil cover.

Soil moisture was determined at nine locations in each bay to a depth of 1.5 m in 6 depth increments.  A mean of the nine-samples per depth was used in further analyses.  Soil moisture was determined three times per year; post-harvest, mid fallow and pre-planting, typically November, February and May.

Bulk density was measured on several occasions using 10 cm diameter cores, and used to convert gravimetric values to a volumetric basis.

To estimate a PAWC for the site, the mean of the five driest and wettest samplings were used. Expressed as mm available to plants.

Grain yield was determined by harvesting 400 m long strips in each bay using a commercial harvester (6m wide).

Evaporation (E) in fallows was determined by difference as E = Rainfall-Runoff -change in soil water over the fallow.

Fallow efficiency was defined as the change in plant available soil water over a fallow period as a percentage of rainfall. Expressed as percentage (%)

A collection of documents contain this data while observations for use in Howleaky are also attached

Total_Cover Total soil cover (%) Wallumbilla catchment study 1983-2000 Data type: Spot observations of soil cover (total) (%). Discontinuous data on dates of observation Total cover (%) is cover from green growing vegetation, crop residue or pasture Used to check crop plant, harvest and tillage conditions are being described by the model Observation dates typically when runoff occurred, soil sampling and site visits Data source: Data was compiled from field notes and hydrologic databases (RRUMS, Hydstra) Queensland Department of Natural Resources 2007 and revised in 2021 This datafile is based on or contains data provided by the State of Queensland (Department of Environment and Resource Management) 2012. (C) The State of Queensland (Department of Environment and Resource Management)

Runoff Sed loss Rainfall, Runoff, Soil Movement, Suspended sediment concentration and load. Wallumbilla catchment study 1982-1997 Data type: Daily Rainfall (mm), Runoff (mm) Soil Movement (t/ha), Suspended sediment concentration (mg/L), Suspended sediment load (t/ha) for four winter crop-fallow treatments (stubble burnt, incorporated, mulch, zero tillage) and a summer crop Continuous data - zero values means no observable variable, not missing data. -continuous data infilled (tilled only) and date corrected Soil loss/erosion represents movement of soil to the bottom of the cultivated plane Suspended sediment concentration was determined at the weir outlet and sediment load calculated from a flow weighted mean concentration and runoff depth (Freebairn and Wockner 1986) Possible timing errors of +-1 day between some observations. Data source: Data was compiled from field notes and hydrologic databases (RRUMS, Hydstra) by David Freebairn and Graeme Wockner, Queensland Department of Natural Resources 2007 and revised in 2021.This datafile is based on or contains data provided by the State of Queensland (Department of Environment and Resource Management) 2012. © The State of Queensland (Department of Environment and Resource Management) 2012.

Plant harvest Yield Plant and harvest dates and wheat yield Wallumbilla catchment study 1983-1999 Data type: Planting and Harvest dates for cropped catchments Plant Flag =9, Harvest flag=99, Wheat yield kg/ha. Discontinuous data on dates of observation Data source: Data was compiled from field notes and hydrologic databases (RRUMS, Hydstra) by David Freebairn and Graeme Wockner, Queensland Department of Natural Resources 2007 and revised in 2021 This datafile is based on or contains data provided by the State of Queensland (Department of Environment and Resource Management) 2012. © The State of Queensland (Department of Environment and Resource Management) 2012.

Soil water Plant available water (PAW) (mm) Wallumbilla catchment study 1982-1999 Data type: Plant available water (PAW) (mm) (0-150cm) organised by tillage treatments PAW based on gravimetric sampling to 150 cm in six increments. Also see "adjusted" values Discontinuous data on dates of observation. On change-over of bays to a new treatment, the site mean PAW for that day is presented on the next day, to indicate a bay change at planting of the next winter crop. Bay number is included in this data Adjusted soil water - PAW adjusted to remove treatment/catchment artefacts e.g. when testing water balance model to estimate fallow water accumulation. PAW is averaged across the site at the start of each fallow period (around Nov each year). Subsequent PAW changes used this common reference value and each individual bay’s subsequent PAW changes referenced to this common start value and date. Meaningful, simulation of soil water require a PAW reset to the start of fallow PAW value to reduce site variability artefacts. Field measured bulk density values used to convert gravimetric values (g/g) to volumetric (cc/cc) Lower limit (LL) or Permanent Wilting Point (PWP) based on minimum soil moisture value for each increment (mean of 9 samples for a date) in each bay except 20.9 g/g% used for surface 0-10 cm layer all bays. Data source: Compiled from field notes and hydrologic databases (RRUMS, Hydstra) by David Freebairn and Graeme Wockner, Queensland Department of Natural Resources 2007 and revised in 2021. This datafile is based on or contains data provided by the State of Queensland (Department of Environment and Resource Management) 2012. © The State of Queensland (Department of Environment and Resource Management) 2012.

9. Simulations

Will simulation reports go in here?

10. Simulation report

Howleaky model assessment - Wallumbilla

Summary

Data from the Wallumbilla runoff and soil erosion catchment study represents 17 years (1983-2000) of monitoring rainfall, runoff, soil water sediment and erosion from 4 contour bay catchments with two tillage treatments (conventional tillage and conservation tillage) after winter cropping and pasture land uses. In this analysis, data for the period 1983-95 is used as this period has the most treatments in common although only eight years of pasture data were available (1990-97). Due to the subtle impacts of tillage management on crops and hydrology, these two treatments are treated as one ): wheat-tilled fallow.

Two versions of the Howleaky model (V5.37.06, McClymont et al. 2011 and http://howleaky.dhmsoftware.com.au/ ) were applied to data from the Wallumbilla catchment study (Freebairn et al 2010) where 17 years of soil water, runoff, sediment and crop data have been collected for a range of tillage treatments under annual wheat cropping, and also buffel grass pasture. The model was not tuned to specific crop management practices such as planting and tillage dates or specific site and management descriptions. A set of generalised soil, crop and tillage regimes were applied.

Overall, the model describes field observations such that the resultant parameter sets can be used with confidence to explore a range of management options (Table 1 and Figure 1). Also, data in Table 1 and Figure 1 demonstrates that the two versions of Howleaky: the stand-alone version (revision 5.49.15) and the www based version provide similar estimations of runoff and soil erosion and that these estimates are in reasonable agreement with field observations.

Table 1 Summary of average annual observations and estimates of runoff and soil erosion at Wallumbilla using the “Cover” and “LAI” crop routines within Howleaky.

| Land management | Observed runoff (mm) | Est. runoff Cover model (mm) | Est. runoff LAI model (mm) | Observed sediment loss (t/ha) | Est. sediment Cover model (t/ha) | Est. sediment LAI model (t/ha) |
| --- | --- | --- | --- | --- | --- | --- |
| Wheat-tilled fallow (1983-1997) | 52.3 | 51.4 | 57.8 | 3.0 | 2.4 | 3.2 |
| Pasture (A) (1990-1995) | 14.1 | 12.8 | - | 0.3 | 0.1 | - |

Figure 1 Observed runoff and erosion vs simulated values using the Howleaky Cover for wheat-cultivated fallow and pasture. (Tilled data 1983-95; pasture 1990-97)

Methodology

Howleaky has approximately 40 variables which can be manipulated to varying degrees: 12-40 values to describe soil water holding capacity; two for evaporation, four for infiltration; two for sediment delivery; a set of curves to describe vegetation (green and dead cover and root depth); and several tillage parameters.

Rather than apply exhaustive model fitting, a pragmatic process was followed to ensure the model reproduced field observations. Once data was assembled, model tuning took 1-2 hours to complete. Little attempt was made to improve model performance if model estimates were in the ballpark of observed values. The Pareto principle was applied ruthlessly. If model output describing the main hydrologic processes survived a quick visual inspection for sensibility (absolute values and trends), parameter sets were accepted. More iterations of parameter values would likely improve the coincidence of modeled and measured values, but the aim was to generate a set of useful parameter values that can be applied with fair confidence in other studies.

Tuning strategy

Build soil file based on observed PAWC (see soil description) – PAW was re-derived using the mean of all bays, lowest set of gravimetric samples (mean of 4 * 4 =36 samples) and measured soil bulk density values. The fact that there was no discernable catena of soil moisture values down the slope or across the site supported averaging across all bays for LL values.

Plant and harvest dates and wheat yields used to tune LAI model for wheat (mid maturity group). A fixed plant date of 25th May was used to avoid uncertainties of variable plant rules. LAI crop model parameters based on literature and field observations.

Spot estimates of total cover were used to check tillage parameters and stubble decay rate. Two tillage regimes created: Chisel and minimum tillage.

Observed and predicted soil water was used to check the model was using water sensibly.

Cumulative predicted and observed runoff were compared and CN and Cover-CN were tuned to achieve O:P ratio near 1.

Cumulative predicted and observed sediment loss were compared and K and delivery ratio were tuned to achieve O:P ratio near unity.

Building the soil profile file

Plant available water (PAW) was measured using gravimetric sampling over a 17-year period. Having the luxury of this length of field observations over both wet and dry seasons should provide a robust estimate of Plant Available Water capacity (PAWC). PAW exceeded 180 mm on 3 occasions in 17 years, and was drier than 20 mm on 4 occasions. While PAWC could be taken as high as 205 mm, a more realistic value of 180 mm PAWC was applied to this dataset. The rationale for this arbitrary decision was that it takes special circumstances to wet up and dry down this soil in the field, but within a model context, the crop will wet up and dry down quicker than is likely to occur in field conditions. See attached document “Soil water variability analysis 2020.docx” for a more detailed discussion on spatial variability in PAW and PAWC.

Figure 2 Observed plant available soil water (PAW) (mm) for winter wheat - summer fallows and a pasture at Wallumbilla. (1983- 1998).

Plant and harvest date, soil cover (crops, pasture and tillage treatments)

Planting and harvest dates (lines at 10 and 100 respectively) and crop cover (%), showing a fixed plant date of 25th May results in reasonable reproduction of crop growth patterns.

Figure 3 Actual planting and harvest dates and static crop growth assumed for the Cover model (left) and LAI model (right). (1983- 1995).

Model performance

Soil water

Profile PAW is predicted with reasonable reliability, at least sufficient to guide the runoff routine in Howleaky but can be underestimated when crop water use should be reduced due to water stress (suspected).

Figure 4 Simulated and observed plant available soil water (mm) for the winter crop, tilled summer fallow (1983-1995) at Wallumbilla.

Figure 4 Simulated and observed plant available soil water (mm) for the winter crop, tilled summer fallow (1983-1995) at Wallumbilla.

Runoff

Figure 5 Simulated and measured cumulative runoff from the winter crop, tilled summer fallow (1982-1998) and pasture (1990- 1998) at Wallumbilla.

Suspended sediment

Figure 6 Simulated and measured cumulative sediment loss(t/ha) from the winter crop, tilled summer fallow (1982-1998) and pasture (1990- 1998) at Wallumbilla.

Crop yield

Wheat yields are simulated with a sensible response to seasonal variation and crop residue cover management (Figure 7, Table 2). This unreplicated study was not designed to compare crop performance between treatments, and treatment effects are relative subtle (compared to other studies) as stubble cover levels are low due to the drier environment of this site.

Figure 7 Observed and simulated (LAI and Cover model) wheat yields at Wallumbilla.

Table 2 Summary of average wheat yield observed and estimated at Wallumbilla (1983-95)

| Observed wheat yield (kg/ha) | Estimated Cover model tillage (kg/ha) | Estimated Cover model con. tillage (kg/ha) | Estimated LAI model tillage (kg/ha) | Estimated LAI model con. tillage (kg/ha) |
| --- | --- | --- | --- | --- |
| 1200 | 1282 | 1367 | 1306 | 1379 |

David Freebairn  davidmaxfreebairn@me.com  +61 408876904

4/02/2021

11. Publications

Freebairn DM, Wockner GH, Hamilton NA, Rowland P. 2009, Impact of soil conditions on hydrology and water quality for a brown clay in the north eastern cereal zone of Australia. Aust. J. Soil Research, 47:389-402

Freebairn DM, Cutajar J. 2011 Howleaky assessment – Wallumbilla ROSL study. Report for “Capturing historic small catchment study (paddock scale) data to support quantification of management impacts on water quality on the Great Barrier Reef (RRRD011).

Freebairn DM, Robinson JB, McClymont DJ, Raine S, Schmidt E, Skowronski V, Eberhard J. 2017. SoilWaterApp – monitoring soil water made easy. in Proceedings of the 18th Australian Society of Agronomy Conference, 24 – 28 September 2017, Ballarat, Australia © 2017. (http://www.agronomyaustraliaproceedings.org/)

Freebairn DM. 2018. Tillage, stubble and zero‐till ‐ understanding the data sets underpinning no‐till farming systems for productivity and resource sustainability. GRDC Updates Dubbo (February), Goondiwindi (March).

Ghahramani A, Freebairn DM, Sena DR, Cutajar JL, Silburn DM (2020) A pragmatic parameterisation and calibration approach to model hydrology and water quality of agricultural landscapes and catchments. Environmental Modelling and Software 130 (2020) 104733

McClymont D, Freebairn DM, Rattray DJ, Robinson JB. Silburn DM, White S. 2010. Howleaky?: Exploring water balance and water quality implication of different land uses. Software V5.37.06 beta.

Serafin, L., Dang, Y., Freebairn D., Rodriguez D. (2019) Chapter 5. Evolution of conservation agriculture in summer rainfall areas. In (Eds J Pratley and J Kirkegaard) “Australian Agriculture in 2020: From Conservation to Automation” pp 65-78 (Agronomy Australia and Charles Sturt University: Wagga Wagga).

Silburn DM, Tolmie PE, Biggs AJW, Whish JPM, French V. 2011.  Deep drainage rates of Grey Vertosols depend on land use in semi-arid subtropical regions of Queensland, Australia Soil Research, 49, 424–438.

Tolmie PE, Silburn DM, Biggs AJW 2011. Deep drainage and soil salt loads in the Queensland Murray–Darling Basin using soil chloride: comparison of land uses Soil Research, 49, 408–423.

12. Other documents

Related documents

These links access a range of miscellaneous outputs from the Wallumbilla study, from progresssive sumamries to details on side experiments (crop establishment, impct of roughness on runoff,statistical analyses) and extension articles in local media. These are added for completeness and as a library for the the site.

Howleaky model assessment 2021

Spatial variability in PAW and PAWC

Emergence trial notes per Bruce radford (pdf)

Water balance synopsis 1994 (Word)

Optimising water capture with tillage 1982-93 (Word)

Options for improving water conservation 1988 (pdf)

Rainfall simulation notes 1991 (Word)

Roughness and rainfall capture 1996 (pdf)

Soil water management in the Maranoa 1982-94 (Word)

Statistical analysis of runoff (cover and roughness) 1990 (Word)

Soil description 1998 (Word)

Water balance study summary 1995 (Word)

Research Gossip 1990 (pdf)

© 2020 - HowLeaky - Privacy